Inter-Micro Communication Arbitration

FDD #AR350A

.

1.	High Level Description	4

2.	Function I/O	5

2.1.	Input Description	5

2.2.	Output Description	5

2.3.	Sub-Function Data Flow	6

2.3.1.	Imc Arbitration Transmit side data flow	6

2.3.2.	Imc Arbitration Receive side data flow	6

2.3.3.	Rolling Counter Handling	7

3.	Design Rationale and Assumptions	14

4.	Sub-Functions	15

4.1.	Sub-Function: ImcArbnInit1	15

4.1.1.	Hardware Related Design	15

4.1.2.	Software Related Design	15

4.2.	Sub-Function: ImcArbnTx	16

4.2.1.	Hardware Related Design	16

4.2.2.	Software Related Design	16

4.3.	Sub-Function: ImcArbnPer1	19

4.3.1.	Hardware Related Design	19

4.3.2.	Software Related Design	19

4.4.	Sub-Function: ImcArbnPer2	19

4.4.1.	Hardware Related Design	19

4.4.2.	Software Related Design	20

4.5.	Sub-Function: ImcArbnPer3	20

4.5.1.	Hardware Related Design	20

4.5.2.	Software Related Design	20

4.6.	Sub-Function: ImcArbnRx	21

4.6.1.	Hardware Related Design	21

4.6.2.	Software Related Design	21

4.7.	Sub-Function: ImcArbnPer4	25

4.7.1.	Hardware Related Design	25

4.7.2.	Software Related Design	25

4.8.	Sub-Function: ImcArbnPer5	26

4.8.1.	Hardware Related Design	26

4.8.2.	Software Related Design	26

4.9.	Sub-Function: ImcArbnPer6	27

4.9.1.	Hardware Related Design	27

4.9.2.	Software Related Design	27

4.10.	Sub-Function:  SetRxSigGroup	29

4.10.1.	Hardware Related Design	29

4.10.2.	Software Related Design	29

4.11.	Sub-Function:  GetTxSigGroup	31

4.11.1.	Hardware Related Design	31

4.11.2.	Software Related Design	31

4.12.	Sub-Function:  GetTxRateGroup	33

4.12.1.	Hardware Related Design	33

4.12.2.	Software Related Design	33

4.13.	Sub-Function:  GetSigImcDataExtdSts_<DATA_TYPE>	35

4.13.1.	Hardware Related Design	35

4.13.2.	Software Related Design	35

4.14.	Sub-Function:  GetSigImcData_<DATA_TYPE>	37

4.14.1.	Hardware Related Design	37

4.14.2.	Software Related Design	37

4.15.	Sub-Function:  RxFrmVldChk	38

4.15.1.	Hardware Related Design	38

4.15.2.	Software Related Design	38

4.16.	Sub-Function:  ImcChResyncHndlg	42

4.16.1.	Hardware Related Design	42

4.16.2.	Software Related Design	42

4.17.	Sub-Function:  CreatSigGroupData	43

4.17.1.	Hardware Related Design	43

4.17.2.	Software Related Design	43

4.18.	Sub-Function:  DecodSigGroupData	44

4.18.1.	Hardware Related Design	44

4.18.2.	Software Related Design	44

4.19.	Sub-Function:  GetBitMask	46

4.19.1.	Hardware Related Design	46

4.19.2.	Software Related Design	46

4.20.	Sub-Function:  GetImcFltParamByte	47

4.20.1.	Hardware Related Design	47

4.20.2.	Software Related Design	47

4.21.	Sub-Function:  VldtRollgCntr	49

4.21.1.	Hardware Related Design	49

4.21.2.	Software Related Design	49

4.22.	Sub-Function:  VldtRollgCntrAlg	51

4.22.1.	Hardware Related Design	51

4.22.2.	Software Related Design	51

4.23.	Sub-Function:  RollgCntrSeqChk	54

4.23.1.	Hardware Related Design	54

4.23.2.	Software Related Design	54

4.24.	Sub-Function:  NoDataHndlg	55

4.24.1.	Hardware Related Design	55

4.24.2.	Software Related Design	55

4.25.	Sub-Function:  EvlSigGroupNeverRxdMissSts	57

4.25.1.	Hardware Related Design	57

4.25.2.	Software Related Design	57

4.26.	Sub-Function:  OvrdSigStsDurgStrtUp	59

4.26.1.	Hardware Related Design	59

4.26.2.	Software Related Design	59

4.27.	Sub-Function:  GetImcSigSts	60

4.27.1.	Hardware Related Design	60

4.27.2.	Software Related Design	60

4.28.	Sub-Function:  NoDataRxd	62

4.28.1.	Hardware Related Design	62

4.28.2.	Software Related Design	62

5.	Timing / Execution Constraints	63

5.1.	Rationale / Comments	63

5.2.	Rates and State Execution	63

6.	Serial Communications Interfaces	63

7.	Additional Information	63

7.1.	Imc Arbitration – Configuration	63

7.1.1.	Hardware Related Design	63

7.1.2.	Software Related Design	63

7.2.	Imc Signal Mapping for Receiving FDDs	69

7.2.1.	Hardware Related Design	69

7.2.2.	Software Related Design	69

7.3.	Enumerations - Signal Status, Extended Signal Status and Signal Source	69

7.3.1.	Hardware Related Design	69

7.3.2.	Software Related Design	69

8.	Revision Record & Change Approval	70

# High Level Description

The Inter-Micro Communication (IMC) Arbitration prepares data for transmission to a complimentary ECU through two redundant communication paths.  On the receive side, the Arbitration reads data from the primary source and determines the data validity.  A validity fault on primary source prompts the IMC Arbitration component to evaluate the same signal from the secondary source.  Faulty signals from the primary source will be replaced with signals from the secondary source, as long as they are valid.  If both data sources are invalid the IMC Arbitration outputs the signal status based on never received, missing and invalid conditions.

# Function I/O

## Input Description

Inputs for this component are provided by IMC signal configurations that are defined based on program requirements. Refer Section 5.23 for more details.

| Input Name | Description |
| --- | --- |
| <Signal1>…<Signal#> | All RTE signals coming from any periodic that are required to be read by IMC Arbitration scheduled runnables. The list of signals can change from program to program based on program dataflow requirements. |

## Output Description

Refer Data Dictionary for details.

## Sub-Function Data Flow

### Imc Arbitration Transmit side data flow

### Imc Arbitration Receive side data flow

### Rolling Counter Handling

General Description:

Rolling counters are used to ascertain sequence of every Signal Group reception via IMC channels.

Rolling counters are embedded in the start byte with length of 5 bits in every frame on both channels. This sequence is generated for each Rate Group separately with range of 0 to 31.  Roll over will happen once the Rolling counter reaches 31.

All Signal Groups defined under a Rate Group will have same value of Rolling Counter in a given cycle of transmission.

On the receiving side Rolling counter evaluation poses following challenges:

- Underlying protocols of Primary and Secondary sources have different data handling methods and hence, at any instant, the data received from any of these channels will be different.

- Underlying protocols of Primary and Secondary sources have different handling in Software which in turn impacts reception hence, at any instant, the data processed will be different.

- Corruption in the channel level causes loss in messages and hence loss in synchronization.

Rolling counter algorithm addresses all these challenges and determines a robust approach to handle these.

Rolling Counter Algorithm description:

The following are addressed by this algorithm:

- Validation of the data sequence of Signal Group based on Rolling counter from Primary and Secondary sources

- Data Sequence validation from IMC channels which are of same or different characteristics.

- Resynchronization of the Rolling counter check during data corruption in the channels.

- Resynchronization of the Rolling counter check when the Rolling counter reference changes ( Roll-over cases)

- Indicates if a Rolling counter fault needs to be reported.

Further details are indicated in the following generic flowchart.

Variables used in the flowchart are,

- DataValid: This is a flag which indicates if valid Data is received from the Communication channel.

- GoodDataSource:This indicates which Communication channel ( Primary or Secondary) has Valid data

- MessageSkipCounter: This is a counter which indicates the number of missed messages in the form of No data or invalid data.

- RollCounterError: Indicates if a rolling counter fault need to be reported

- CounterThreshold:This is a value related to the typical amount of latency in data transmission in the communication channel.

- ChannelSwitchDelay:This is a value related to the dynamics of the redundant communication channels. This indicates the typical delay in a message reception between the communication channels at any instant.

- PreviousRolling Counter:This is the value of the rolling counter of the previously stored valid message.

- RollCounterResyncCounterPrimary: This is a counter which indicates the number of times data is missed because of only Rolling Counter issue on primary channel.

- RollCounterResyncCounterSecondary: This is a counter which indicates the number of times data is missed because of only Rolling Counter issue on Secondary channel.

- ResyncThreshold: This is the number of consecutive Rolling counter issues after which it could be assumed that either of the MCUs have lot synchronization of rolling counter, and hence a resync has to happen.

- RollCounterPrimaryResync – Previous rolling counter value received during resync period on Primary channel

- RollCounterSecondaryResync – Previous rolling counter value received during resync period on Secondary channel

- PrimaryResyncActive –  Indicates whether Resync is active on Primary channel

- SecondaryResyncActive –  Indicates whether Resync is active on Secondary channel.

In short, the Rolling counter algorithm behaves as follows,

- Data is considered valid in the following cases,

  - For consecutive messages from same Comm. channel, if the new Rolling counter falls within the range

(ExpectedRollCntrValue  – LowerLimit)  <= CurrentRolling Counter <= (ExpectedRollCntrValue  + CounterThreshold)

Where ExpectedRollCntrValue = PreviousRolling Counter + MessageSkipCounter + 1,

LowerLimit = CounterThreshold if it is lesser than MessageSkipCounter else, LowerLimit = MessageSkipCounter

  - For consecutive messages from different Comm. channel, if the new RL falls within a range specified by

(ExpectedRollCntrValue – LowerLimit)  <= CurrentRolling Counter <= (ExpectedRollCntrValue  + (CounterThreshold + ChannelSwitchDelay))

Where ExpectedRollCntrValue = PreviousRolling Counter + MessageSkipCounter + 1,

LowerLimit = CounterThreshold + ChannelSwitchDelay if it is lesser than MessageSkipCounter else, LowerLimit = MessageSkipCounter

- Rolling Counter error will not be set in the following cases,

  - For consecutive messages from same Comm. channel, if the new Rolling counter falls within the range

(ExpectedRollCntrValue  – CounterThreshold)  <= CurrentRolling Counter <= (ExpectedRollCntrValue  + CounterThreshold)

Where ExpectedRollCntrValue = PreviousRolling Counter + MessageSkipCounter + 1

  - For consecutive messages from different Comm. channel, if the new RL falls within a range specified by

(ExpectedRollCntrValue – (CounterThreshold + ChannelSwitchDelay))  <= CurrentRolling Counter <= (ExpectedRollCntrValue  + (CounterThreshold + ChannelSwitchDelay))

Where ExpectedRollCntrValue = PreviousRolling Counter + MessageSkipCounter + 1

- On every skip of message (includes missing, CRC error and No data) MessageSkipCounter is incremented. The next rolling counter is expected to have a value bigger than the previous rolling counter value by this counter amount. Any active resynchronization will be cleared as next rolling counter value will not be just next to currently stored value.

- If consecutive ResyncThreshold amount of rolling counter issues occur (cases when old or invalid CurrentRollingCounter detected) in a channel, then it is assumed that either of the MCUs have lot synchronization of rolling counter on the particular channel, and hence a resynchronization is triggered.

- Following pseudo code explains resynchronization logic on Primary channel. Same is applicable for Secondary channel as well. This logic will start run when old or invalid CurrentRollingCounter detected in a channel. Note that at any point of time, Resync will be active for only one channel.

If((Channel == Primary) &&  (SecondaryResyncActive == FALSE)) /* Secondary Resync not active */

{

If(RollCounterPrimaryResync == 0xFF) /* First time Rolling counter fault detected */

{

RollCounterResyncCounterPrimary ++ /* Update Resync counter */

RollCounterPrimaryResync = CurrentRollingCounter /* Update current rolling counter */

PrimaryResyncActive = TRUE /*Activate primary resync */

}

Else

{

If(CurrentRollingCounter == (RollCounterPrimaryResync+1) /* Check new rolling counter is just next to current previous rolling counter */

{

RollCounterResyncCounterPrimary ++ /* Good to proceed resync. Increment Resync counter to reach threshold. */

RollCounterPrimaryResync = CurrentRollingCounter /* Update Rolling counter */

}

Else /* Rolling counter is not next value. Abort Resync. Try again */

{

RollCounterResyncCounterPrimary  = 0

RollCounterPrimaryResync = 0xFF

PrimaryResyncActive = FALSE

}

}

}

When RollCounterResyncCounterPrimary reaches ResyncThreshold, Rolling counter  and data will be declared as valid.

- For rolling counter evaluation within the same Comm. Channel, a latency value of CounterThreshold is used. For rolling counter evaluation within different Comm. Channel, an additional latency value of ChannelSwitchDelay is used.

Following flow-charts indicates overall rolling counter handling.

Rolling counter algorithm spreads across following functions:

- ImcArbnInit1 – Initialization of Signal Group Resync Rolling counter buffer and Signal Group data source buffer

- ImcArbnRx –  Resync and Skip counter handling during no data situation.

- RxFrmVldChk - Resync and Skip counter handling

- VldtRollgCntr – Validate rolling counter

- VldtRollgCntrAlg – Rolling counter algorithm implementation

- RollgCntrSeqChk – Valid Rolling counter interval checking

- NoDataHndlg – Rolling counter handling when No data received on both channels

- ImcChResyncHndlg – Resync logic

# Design Rationale and Assumptions

- Imc Arbitration Receive function designed based on the assumption that first byte and last byte in a valid Signal Group is always a non-zero value.

- Signal Group data received by IMC Arbitration component considered as valid if CRC and rolling counters found to be good. This means, IMC Arbitration component is not designed to evaluate quality of individual signals packed in a Signal Group. The signal status provided by this component is just mere expression of quality of reception. It is assumed that evaluation of quality of Signal would be carried-out by respective sending/receiving components.

- Imc Arbitration component doesn’t use fault threshold tracking mechanism implemented by Diagnostics Manager due to the fact that this component is currently assigned with one NTC (NTC 0x3F) for logging all frame faults and these faults are detected across three Rx periodics (ImcArbnPer4, ImcArbnPer5 and ImcArbnPer6). Diag Manager offers tracking mechanism only for a single periodic. This component keeps track of faulty frames using PIM variables and logs/clears the NTC in 100ms Rx periodic (ImcArbnPer6).

- Imc Arbitration component doesn’t care about availability of data bandwidth to transmit /receive all configured Signal Groups. It allows the user to configure up to 255 Signal Groups. Assumption is that optimal use of IMC channels would be ensured during system design.

- Imc Arbitration component doesn’t care about channel type of primary source or secondary source. This component should work independent of type of channel as long as the frame format of a Signal Group followed.

- Imc Arbitration Tx functionality provides interfaces to get transmit data for individual Signal Group or all signal groups configured under given Rate Group. It is up to the primary and secondary source Tx layers to decide how they would like to get the Tx data from IMC Arbitration component, considering all relevant requirements.

- It is mandatory to transmit all Signal Groups on primary source channel. Transmission of particular Signal Group on secondary source channel is optional and that need to be specified during configuration.

- Tx / Rx processing for particular Rate Group will happen only if it has minimum one Signal Group. Otherwise, evaluation of Tx / Rx processing of said Rate Group will be skipped.

- This component provides Signal status definition in two versions: basic and extended. While basic signal status indicates No Data, Valid and Invalid cases, extended signal status provides status of startup, Signal Group never received, Signal Group missing, Signal Group availability on only one channel or on both channels. Following table indicates relation between these two status:

- Signal Status IMCARBNRXEXTDSTS_STRTG / IMCARBNRXSTS_NODATA returned to IMC Signal users until initial start-up time completion even if integrity of received signal data found to be good. This is to ensure that the availability of properly calculated values in RTE by the components which are transmitting signals via IMC.

- Data consistency across the component is ensured by exclusive area access. Following are the places where exclusive area access provided:

  - Reading from / Writing into  Tx Buffer

  - Reading from / Writing into  Rx buffer

  - Writing to Received signal data buffer, Status buffer and Signal source buffers.

- This component provides two server runnables to copy data buffer during transmission: one for Rate Group level (provides all Signal Groups configured under given Rate Group) and another for Signal Group level (provides all Signals configured under given Signal Group). It is up to the respective channel drivers to decide which one to be invoked depending on buffer handling scheme.

- Primary and Secondary source receive data buffers are defined with same size even though, transmission of Signal Groups on Secondary source is optional. It is done to avoid complex logics in buffer handling. For e.g if SignalGroupA is configured to be PrimaryOnly, then there is no need to allocate buffer space in the Secondary source buffer for SignalGroupA. But, the design allocates buffer SignalGroupA as well in the Secondary source buffer to keep common buffer definition and access logic.

- The server runnables defined in this component designed based on the assumption that it will be invoked only from the tasks which runs at the rate of 2ms or greater than that.

# Sub-Functions

## Sub-Function: ImcArbnInit1

### Hardware Related Design

None

### Software Related Design

| ImcArbnInit1 |  |
| --- | --- |
| Function scope | Global, Init function |
| Description | Initialized Signal Group and Signal related buffers. Gets start time of the component and saves it in a PIM |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Reads / Writes to following variables in functions |
|  | SigGroupNeverRxd |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
|  | RxdSigDataExtdSts |
|  | RxdSigDataSrc |
|  | SigGroupDataSrc |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | Accesses following configuration data |
|  | IMCARBN_TOTALNROFSIG_CNT_U16 |
|  | IMCARBN_TOTALNROFSIGGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | Init Task |

## Sub-Function: ImcArbnTx

### Hardware Related Design

None

### Software Related Design

| ImcArbnTx |  |
| --- | --- |
| Function scope | Local |
| Description | 1) This function collects signal data for all Signal Groups configured in the ImcArbn_SigGroupConfig_Rec for given Rate Group and packs each signal to the specified start bit in each Signal Group as defined in the configuration. 2) A Pattern Identification flag, Rolling Counter, Signal Group ID, CRC, Compliment Pattern ID and Compliment Rolling Counter are added for each Signal Group. 3) The resulting signal group is put into a transmit data buffer for for the GetTxSigGroup or GetTxRateGroup client calls to access. |
| Parameters | Requires following parameters |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
| Return value | None |
| Affected static and global variables | Writes to following variables |
|  | TxBuf |
|  | RollgCntr |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to update Transmit data buffer |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | CreatSigGroupData |
| External Interface call | Calls following external function in the sub-functions |
|  | Crc_CalculateCRC8 |
| Called by | Called by following functions only when atleast one Signal Group is configured for particular Rate Group e.g if no Signal Group configured for 100ms Rate Group then this function will not get called from ImcArbnPer3 |
|  | ImcArbrPer1 |
|  | ImcArbrPer2 |
|  | ImcArbrPer3 |

## Sub-Function: ImcArbnPer1

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer1 |  |
| --- | --- |
| Function scope | 2ms Periodic, Global |
| Description | Frames transmit data for all Signal Groups configured under 2ms Rate Group and stores in the Transmit data buffer. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Writes to following variables |
|  | TxBuf |
|  | RollgCntr |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to update Transmit data buffer |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | Calls following function only when number of Signal Groups configured under the 2ms Rate Group is NOT zero (IMCARBN_NROF2MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnTx |
| External Interface call | Calls following external function |
|  | Crc_CalculateCRC8 |
| Called by | 2ms Task |

## Sub-Function: ImcArbnPer2

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer2 |  |
| --- | --- |
| Function scope | 10ms Periodic, Global |
| Description | Frames transmit data for all Signal Groups configured under 10ms Rate Group and stores in the Transmit data buffer. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Writes to following variables |
|  | TxBuf |
|  | RollgCntr |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to update Transmit data buffer |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | Calls following function only when number of Signal Groups configured under the 10ms Rate Group is NOT zero (IMCARBN_NROF10MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnTx |
| External Interface call | Calls following external function in the sub-functions |
|  | Crc_CalculateCRC8 |
| Called by | 10ms Task |

## Sub-Function: ImcArbnPer3

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer3 |  |
| --- | --- |
| Function scope | 100ms Periodic, Global |
| Description | Frames transmit data for all Signal Groups configured under 100ms Rate Group and stores in the Transmit data buffer. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Writes to following variables |
|  | TxBuf |
|  | RollgCntr |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to update Transmit data buffer |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | Calls following function only when number of Signal Groups configured under the 100ms Rate Group is NOT zero (IMCARBN_NROF100MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnTx |
| External Interface call | Calls following external function in the sub-functions |
|  | Crc_CalculateCRC8 |
| Called by | 100ms Task |

## Sub-Function: ImcArbnRx

### Hardware Related Design

None

### Software Related Design

| ImcArbnRx |  |
| --- | --- |
| Function scope | Local |
| Description | 1) This sub-function receives signal groups from two Imc Channels: Primary Source and Secondary Source. It first validates the data from the primary source and if there is any fault found with the Signal Group data, it will switch to the secondary source for that signal. 2) If the Signal Group data has been sourced from primary source, it checks availability of the Signal Group data received via secondary source. If the signal group is available in secondary source buffer, then it sets respective signals defined under signal group to IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP. Otherwise it sets the Signal status to IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP. 3) If the signal group data received on primary source found to be invalid, then it switches to secondary source data buffer and evaluates it. If valid, the signal status is set to IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP. Otherwise set to IMCARBNRXEXTDSTS_DATAINVLD. 4) If the Signal Group is not received in any of the channels, then it evaluates missing and never received conditions and sets signal status to any of the following depends on the conditions: IMCARBNRXEXTDSTS_NEVERRXD, IMCARBNRXEXTDSTS_MISS, IMCARBNRXEXTDSTS_PREVDATA 5) If initial start-up timer is not complete, then signal status is set to IMCARBNRXEXTDSTS_STRTG irrespective of above evaluations. 6) There are a total of two Rx buffers are defined as input to this function: one for primary source, another one for secondary source. 7) There are three buffers (Signal Data, Signal Status, Signal Source) defined to serve signal data requests from the components which uses signals received via IMC Arbitration component. This function puts processed values into these buffers. 8) PIM variables are used to track frame faults and log NTC based on threshold. 9) Following validation steps are carried out to declare received Signal Group data as valid / invalid: a)No Data Received check b)CRC Check c) Rolling counter check |
| Parameters | Requires following parameters |
|  | FrmFltCntr - Pointer to the frame fault counters for the given Rate Group where frame faults to be accounted. Type: Ary2D_u8_2_2* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | IniTiOutChkCmpl - Initial time-out completion status Type: boolean TRUE – Time-out completed FALSE – Time-out not completed |
| Return value | void |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | SigGroupMissCntr |
|  | SigGroupNeverRxd |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | PrevRollgCntrRxd |
|  | PrimSrcRxBuf |
|  | SecdrySrcRxBuf |
|  | RxdSigDataSrc |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | SecdrySrcResyncCntr |
|  | ResyncOnPrimActv |
|  | ResyncOnSecdryActv |
|  | SigGroupDataSrc |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
|  | FrmFltCntr2MilliSec |
|  | FrmFltCntr10MilliSec |
|  | FrmFltCntr100MilliSec |
| Affected inter-runnable variable | Reads following interunnable variable |
|  | IniTiOutChkCmpl |
| Exclusive area access | Needed to read received Signal Group data buffers and to write Imc Signal data, Signal Status and Signal source buffers. |
| Configuration access | Accesses following configuration data in the sub-functions call |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
|  | MISSSIGGROUPTIOUT_CNT_U08 |
| Calibration data access | Accesses following calibration value in the sub-functions call |
|  | ImcArbnFrmFltThd |
| Called local functions | Calls following local function |
|  | RxFrmVldChk |
|  | DecodSigGroupData |
|  | NoDataHndlg |
|  | OvrdSigStsDurgStrtUp |
|  | NoDataRxd |
| External Interface call | Calls following external function |
|  | Crc_CalculateCRC8 |
| Called by | Called by following functions only when atleast one Signal Group is configured for particular Rate Group e.g if no Signal Group configured for 100ms Rate Group then this function will not get called from ImcArbnPer6. |
|  | ImcArbrPer4 |
|  | ImcArbrPer5 |
|  | ImcArbrPer6 |

## Sub-Function: ImcArbnPer4

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer4 |  |
| --- | --- |
| Function scope | 2ms Periodic, Global |
| Description | Processes received Signal Group data via IMC Channels for all Signal Groups configured under 2ms Rate Group and stores validated Signal data, Signal Status and Signal source in respective buffers. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | SigGroupMissCntr |
|  | SigGroupNeverRxd |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | PrevRollgCntrRxd |
|  | PrimSrcRxBuf |
|  | SecdrySrcRxBuf |
|  | FrmFltCntr2MilliSec |
|  | RxdSigDataSrc |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | SecdrySrcResyncCntr |
|  | ResyncOnPrimActv |
|  | ResyncOnSecdryActv |
|  | SigGroupDataSrc |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
| Affected inter-runnable variable | Reads following inter-runnable variable |
|  | IniTiOutChkCmpl |
| Exclusive area access | Needed to read received Signal Group data buffers and write to Imc Signal data, Signal Status and Signal source buffers |
| Configuration access | Accesses following configuration data in the sub-functions call |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
|  | MISSSIGGROUPTIOUT_CNT_U08 |
| Calibration data access | Accesses following calibration value in the sub-functions call |
|  | ImcArbnFrmFltThd |
| Called local functions | Calls following function only when number of Signal Groups configured under the 2ms Rate Group is NOT zero (IMCARBN_NROF2MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnRx |
| External Interface call | Calls following external function |
|  | Crc_CalculateCRC8 |
| Called by | 2ms Task |

## Sub-Function: ImcArbnPer5

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer5 |  |
| --- | --- |
| Function scope | 10ms Periodic, Global |
| Description | Processes received Signal Group data via IMC Channels for all Signal Groups configured under 10ms Rate Group and stores validated Signal data, Signal Status and Signal source in respective buffers. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | SigGroupMissCntr |
|  | SigGroupNeverRxd |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | PrevRollgCntrRxd |
|  | PrimSrcRxBuf |
|  | SecdrySrcRxBuf |
|  | FrmFltCntr10MilliSec |
|  | RxdSigDataSrc |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | SecdrySrcResyncCntr |
|  | ResyncOnPrimActv |
|  | ResyncOnSecdryActv |
|  | SigGroupDataSrc |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
| Affected inter-runnable variable | Reads following inter-runnable variable |
|  | IniTiOutChkCmpl |
| Exclusive area access | Needed to read received Signal Group data buffers and write to Imc Signal data, Signal Status and Signal source buffers |
| Configuration access | Accesses following configuration data in the sub-functions call |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
|  | MISSSIGGROUPTIOUT_CNT_U08 |
| Calibration data access | Accesses following calibration value in the sub-functions call |
|  | ImcArbnFrmFltThd |
| Called local functions | Calls following function only when number of Signal Groups configured under the 10ms Rate Group is NOT zero (IMCARBN_NROF10MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnRx |
| External Interface call | Calls following external function |
|  | Crc_CalculateCRC8 |
| Called by | 10ms Task |

## Sub-Function: ImcArbnPer6

### Hardware Related Design

None

### Software Related Design

| ImcArbrPer6 |  |
| --- | --- |
| Function scope | 100ms Periodic, Global |
| Description | 1. Evaluates initial start-up time out and sets IniTiOutChkCmpl flag once start-up exceeded ECU initialization timeout ImcArbnEcuIninTiOutThd. IniTiOutChkCmpl would be used across all Rx periodics to set signal status IMCARBNRXEXTDSTS_STRTG. 2. Processes received Signal Group data via IMC Channels for all Signal Groups configured under 100ms Rate Group and stores validated Signal data, Signal Status and Signal source in respective buffers. 3. Logs NTCNR_0X03F if any of Frame Fault counters exceeds ImcArbnFrmFltThd. |
| Parameters | None |
| Return value | None |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | Read and Write to following variables in sub-functions |
|  | SigGroupMissCntr |
|  | SigGroupNeverRxd |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | PrevRollgCntrRxd |
|  | PrimSrcRxBuf |
|  | SecdrySrcRxBuf |
|  | FrmFltCntr100MilliSec |
|  | RxdSigDataSrc |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | SecdrySrcResyncCntr |
|  | ResyncOnPrimActv |
|  | ResyncOnSecdryActv |
|  | SigGroupDataSrc |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
| Affected inter-runnable variable | Reads /Writes following inter-runnable variable |
|  | IniTiOutChkCmpl |
| Exclusive area access | Needed to read received Signal Group data buffers and write to Imc Signal data, Signal Status and Signal source buffers |
| Configuration access | Accesses following configuration data in the sub-functions call |
|  | SIGGROUPCONFIG_REC |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
|  | MISSSIGGROUPTIOUT_CNT_U08 |
| Calibration data access | Accesses following calibration value in the sub-functions call |
|  | ImcArbnEcuIninTiOutThd |
|  | ImcArbnFrmFltThd |
| Called local functions | Calls following local function |
|  | GetImcFltParamByte |
|  | Calls following function only when number of Signal Groups configured under the 100ms Rate Group is NOT zero (IMCARBN_NROF100MILLISECRATEGROUP_CNT_U08 != 0) |
|  | ImcArbnRx |
| External Interface call | Calls following external function |
|  | GetTiSpan100MicroSec32bit |
|  | SetNtcSts |
|  | Crc_CalculateCRC8 |
| Called by | 100ms Task |

## Sub-Function:  SetRxSigGroup

### Hardware Related Design

None

### Software Related Design

| SetRxSigGroup |  |
| --- | --- |
| Function scope | Server Runnable, Global |
| Description | This server function is called by primary / secondary source Rx layers to store signal group data that have been received over IMC channels. |
| Parameters | Requires following parameters |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | DataSrc - Imc Channel ID from which the server runnable called Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | Buf - Pointer to the received Signal Group data buffer Type: Ary1D_u8_8* Valid Range: 0x00000001 to 0xFFFFFFFF |
| Return value | Returns following value |
|  | ErrCod - Informs the caller about status of the operation Type: Std_ReturnType Range: E_OK (0) E_NOT_OK (1) |
| Affected static and global variables | Writes to following variables in sub-functions |
|  | PrimSrcRxBuf |
|  | SecdrySrcRxBuf |
|  | SigGroupNeverRxd |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to write primary and secondary source received data buffers. |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Primary and Secondary Source drivers |

## Sub-Function:  GetTxSigGroup

### Hardware Related Design

None

### Software Related Design

| GetTxSigGroup |  |
| --- | --- |
| Function scope | Server Runnable, Global |
| Description | This server function is called by primary / secondary source Tx layers to get signal group data to transmit over IMC channels. |
| Parameters | Requires following parameters |
|  | SigGroupId - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | Buf - Pointer to the buffer where transmit data to be copied Type: uint8* Valid Range: 0x00000001 to 0xFFFFFFFF |
| Return value | Returns following value |
|  | ErrCod - Informs the caller about status of the operation Type: Std_ReturnType Range: E_OK (0) E_NOT_OK (1) |
| Affected static and global variables | Reads following variable |
|  | TxBuf |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to read from Tx data buffer. |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Primary and Secondary Source drivers |

## Sub-Function:  GetTxRateGroup

### Hardware Related Design

None

### Software Related Design

| GetTxRateGroup |  |
| --- | --- |
| Function scope | Server Runnable, Global |
| Description | This server function is called by primary / secondary source Tx layers to get transmit data of all Signal Groups configured under given Rate Group. |
| Parameters | Requires following parameters |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | Buf - Pointer to the buffer where transmit data to be copied Type: uint8* Valid Range: 0x00000001 to 0xFFFFFFFF |
| Return value | Returns following value |
|  | ErrCod - Informs the caller about status of the operation Type: Std_ReturnType Range: E_OK (0) E_NOT_OK (1) |
| Affected static and global variables | Reads following variable |
|  | TxBuf |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to read from Tx data buffer. |
| Configuration access | Accesses following configuration data |
|  | RATEGROUPOFFS_CNT_U08 |
|  | NRSIGGROUPINRATEGROUP_CNT_U08 |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Primary and Secondary Source drivers |

## Sub-Function:  GetSigImcDataExtdSts_<DATA_TYPE>

### Hardware Related Design

None

### Software Related Design

| GetSigImcDataExtdSts_<DATA_TYPE> |  |
| --- | --- |
| Function scope | Server Runnable, Global |
| Description | 1) This server function is called by application components to get particular Signal data received over IMC Channels, Extended Signal Status and Signal Source. 2) There are 8 Server runnables defined to get signal data with extended signal status received via Imc channels each of which called based on received signal data type. Place holder <DATA_TYPE> can be any of the following: f32 (float32), u32 (uint32), s32 (sint32), u16 (uint16), s16 (sint16), u08 (uint8), s08 (sint8), logl (boolean). |
| Parameters | Requires following parameters |
|  | SigId - IMC Signal Id provided by IMC Arbitration Configuration Type: uint16 Range: 0 to 65535 |
|  | Data - Pointer to the signal data buffer where signal data to be copied Type: uint8* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | Sts - Pointer to signal status buffer where signal status to be copied Type: ImcArbnRxSts1* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | DataSrc - Imc Channel ID from which the signal data received Type: ImcArbnRxDataSrc1* Valid Range: 0x00000001 to 0xFFFFFFFF |
| Return value | Returns following value |
|  | ErrCod - Informs the caller about status of the operation Type: Std_ReturnType Range: E_OK (0) E_NOT_OK (1) |
| Affected static and global variables | Reads following variable |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | RxdSigDataSrc |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to read from Signal data, Signal status and Signal Source buffers |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Application components which needs signal data received over IMC Channels |

## Sub-Function:  GetSigImcData_<DATA_TYPE>

### Hardware Related Design

None

### Software Related Design

| GetSigImcData_<DATA_TYPE> |  |
| --- | --- |
| Function scope | Server Runnable, Global |
| Description | 1) This server function is called by application components to get particular Signal data received over IMC Channels and Signal Status. 2) There are 8 Server runnables defined to get signal data with signal status received via Imc channels each of which called based on received signal data type. Place holder <DATA_TYPE> can be any of the following: f32 (float32), u32 (uint32), s32 (sint32), u16 (uint16), s16 (sint16), u08 (uint8), s08 (sint8), logl (boolean). |
| Parameters | Requires following parameters |
|  | SigId - IMC Signal Id provided by IMC Arbitration Configuration Type: uint16 Range: 0 to 65535 |
|  | Data - Pointer to the signal data buffer where signal data to be copied Type: uint8* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | Sts - Pointer to signal status buffer where signal status to be copied Type: ImcArbnRxSts1* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | DataSrc - |
| Return value | Returns following value |
|  | ErrCod - Informs the caller about status of the operation Type: Std_ReturnType Range: E_OK (0) E_NOT_OK (1) |
| Affected static and global variables | Reads following variable |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to read from Signal data and Signal status buffers |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | GetImcSigSts |
| External Interface call | None |
| Called by | Application components which needs signal data received over IMC Channels |

## Sub-Function:  RxFrmVldChk

### Hardware Related Design

None

### Software Related Design

| RxFrmVldChk |  |
| --- | --- |
| Function scope | Local |
| Description | Evaluates validity of received Signal Group data based on CRC and Rolling counter values and returns status that indicates whether given Signal Group data, rolling counter values are valid or not. In addition to that it tracks CRC and Rolling counter frame faults per Rate Group per fault type (CRC / Rolling counter) using Frame Fault counters. This function also tracks Skip counters and Resync counters for Rolling counter evaluation. |
| Parameters | Requires following parameters |
|  | DataBuf - Pointer to the received Signal Group data buffer Type: uint8* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | FrmFltCntr - Pointer to the frame fault counters for the given Rate Group where frame faults to be accounted. Type: Ary2D_u8_2_2* Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | SigGroupDataSrc- Imc Channel ID on which Signal Group data received Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | PrimSrcOnlySigGroup - Flag to indicate whether the given Signal Group is only transmitted on Primary Source or No data received on secondary source. Type: boolean TRUE – Primary Source only Signal Group or No data received on secondary source FALSE – Transmtted on both Primary and Secondary sources |
|  | IniTiOutChkCmpl - Initial time-out completion status Type: boolean TRUE – Time-out completed FALSE – Time-out not completed |
| Return value | Returns following value |
|  | FrmSts - Informs the caller about validity of Rolling counter and Data Type: ImcArbnRxRollgCntrSts1 Range: ROLLGCNTRVLDNEWDATA (0) ROLLGCNTRVLDOLDDATA (1) ROLLGCNTRINVLD (2) |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | PrimSrcRollgCntrResync |
|  | ResyncOnPrimActv |
|  | SecdrySrcResyncCntr |
|  | SecdrySrcRollgCntrResync |
|  | ResyncOnSecdryActv |
|  | FrmFltCntr2MilliSec |
|  | FrmFltCntr10MilliSec |
|  | FrmFltCntr100MilliSec |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | Crc_CalculateCRC8 |
|  | ImcChResyncHndlg |
|  | VldtRollgCntr |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnRx |

## Sub-Function:  ImcChResyncHndlg

### Hardware Related Design

None

### Software Related Design

| ImcChResyncHndlg |  |
| --- | --- |
| Function scope | Local |
| Description | Runs Resynchronization logic for Primary and Secondary sources |
| Parameters | Requires following parameters |
|  | SigGroupDataSrc- Imc Channel ID on which Signal Group data received Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | PrsntRollgCntr - Present Rolling counter received for the Signal Group Type: uint8 Range: 0 to 31 |
| Return value | None |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | PrimSrcResyncCntr |
|  | PrimSrcRollgCntrResync |
|  | ResyncOnPrimActv |
|  | SecdrySrcResyncCntr |
|  | SecdrySrcRollgCntrResync |
|  | ResyncOnSecdryActv |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | RxFrmVldChk |

## Sub-Function:  CreatSigGroupData

### Hardware Related Design

None

### Software Related Design

| CreatSigGroupData |  |
| --- | --- |
| Function scope | Local |
| Description | Collect Signal data for all signals configured under given Signal Group, combine all into a uint32 data based on configuration and return to the caller. |
| Parameters | Requires following parameters |
|  | SigGroup_T_rec - Pointer to the Signal Group configuration data Type: SigGroupRec1 const * Valid Range: 0x00000001 to 0xFFFFFFFF |
| Return value | Returns following value |
|  | RetData - Combined four byte data for given Signal Group configuration Type: uint32 Range: 0x00000000 to 0xFFFFFFFF |
| Affected static and global variables | None |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
| Calibration data access | None |
| Called local functions | None |
| Called local functions | Calls following local function |
|  | Calls Signal access wrapper functions for each signal configured under given Signal Group |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnTx |

## Sub-Function:  DecodSigGroupData

### Hardware Related Design

None

### Software Related Design

| DecodSigGroupData |  |
| --- | --- |
| Function scope | Local |
| Description | Split given uint32 data into Signal data according to the configuration for given Signal Group, store Signal values in Rx Signal Data buffer, updates Signal Status and Signal Source. |
| Parameters | Requires following parameters |
|  | SigGroup_T_rec - Pointer to the Signal Group configuration data Type: SigGroupRec1 const * Valid Range: 0x00000001 to 0xFFFFFFFF |
|  | SigGroupRxData - Combined four bytes received data for given Signal Group configuration Type: uint32 Range: 0x00000000 to 0xFFFFFFFF |
|  | RxSigExtdSts1 - Extended Signal Status Type: ImcArbnRxExtdSts1 Range: IMCARBNRXEXTDSTS_NEVERRXD (0U) IMCARBNRXEXTDSTS_MISS (1U) IMCARBNRXEXTDSTS_DATAINVLD (2U) IMCARBNRXEXTDSTS_PREVDATA (3U) IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP (4U) IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP (5U) IMCARBNRXEXTDSTS_STRTG (6U) All other values are invalid |
|  | RxDataSrc - Imc Channel ID from which the Signal Group data received Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
| Return value | None |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | RxdSigData |
|  | RxdSigDataExtdSts |
|  | RxdSigDataSrc |
| Affected inter-runnable variable | None |
| Exclusive area access | Needed to write Imc Signal data, Signal Status and Signal source buffers. |
| Configuration access | Accesses following configuration data |
|  | ImcArbn_SigGroupConfig_Rec |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | GetBitMask |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbrRx |

## Sub-Function:  GetBitMask

### Hardware Related Design

None

### Software Related Design

| GetBitMask |  |
| --- | --- |
| Function scope | Local |
| Description | Returns bitmask for requested number of bits. |
| Parameters | Requires following parameters |
|  | NrOfBits - Number of bits required to be set in the return value Type: uint8 Range: 1 to 32 |
| Return value | Returns following value |
|  | BitMask - Bit mask for given number of bits Type: uint32 Range: 0x00000000 to 0xFFFFFFFF |
| Affected static and global variables | None |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | CreatSigGroupData |
|  | DecodSigGroupData |

## Sub-Function:  GetImcFltParamByte

### Hardware Related Design

None

### Software Related Design

| GetImcFltParamByte |  |
| --- | --- |
| Function scope | Local |
| Description | Evaluates Frame faults counters and returns IMC Arbitration fault parameter byte to log / clear NTC 0x3F (IMC Arbitration fault). Bits which are corresponding to particular fault that exceeds threshold limit will be set in the return value. Refer below picture for details of bit mapping in the parameter byte |
| Parameters | void |
| Return value | Returns following value |
|  | FltBitMask - Bit mask for given number of bits Type: uint8 Range: Bitmask PRIMSRCCRCFLT_CNT_U08 (0x01U) PRIMSRCROLLGCNTRFLT_CNT_U08 (0x02U) SECDRYSRCCRCFLT_CNT_U08 (0x04U) SECDRYSRCROLLGCNTRFLT_CNT_U08 (0x08U) All other bits are unused. |
| Affected static and global variables | Reads following variables |
|  | FrmFltCntr2MilliSec |
|  | FrmFltCntr10MilliSec |
|  | FrmFltCntr100MilliSec |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | Accesses following calibration value in the sub-functions call |
|  | ImcArbnFrmFltThd |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnPer6 |

## Sub-Function:  VldtRollgCntr

### Hardware Related Design

None

### Software Related Design

| VldtRollgCntr |  |
| --- | --- |
| Function scope | Local |
| Description | Validates Rolling Counter |
| Parameters | Requires following parameters |
|  | SigGroupDataSrc- Imc Channel ID on which Signal Group data received Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | PrsntRollgCntr- Present Rolling counter value received for the given Signal Group Type: uint8 Range: 0 to 31 |
| Return value | Returns following value |
|  | FrmSts - Informs the caller about validity of Rolling counter and Data Type: ImcArbnRxRollgCntrSts1 Range: ROLLGCNTRVLDNEWDATA (0) ROLLGCNTRVLDOLDDATA (1) ROLLGCNTRINVLD (2) |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | PrimSrcResyncCntr |
|  | PrimSrcRollgCntrResync |
|  | ResyncOnPrimActv |
|  | SecdrySrcResyncCntr |
|  | SecdrySrcRollgCntrResync |
|  | ResyncOnSecdryActv |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | VldtRollgCntrAlg |
| External Interface call | None |
| Called by | Called by following functions |
|  | RxFrmVldChk |

## Sub-Function:  VldtRollgCntrAlg

### Hardware Related Design

None

### Software Related Design

| VldtRollgCntrAlg |  |
| --- | --- |
| Function scope | Local |
| Description | Implements Rolling counter algorithm |
| Parameters | Requires following parameters |
|  | SigGroupDataSrc- Imc Channel ID on which Signal Group data received Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | PrsntRollgCntr- Present Rolling counter value received for the given Signal Group Type: uint8 Range: 0 to 31 |
| Return value | Returns following value |
|  | FrmSts - Informs the caller about validity of Rolling counter and Data Type: ImcArbnRxRollgCntrSts1 Range: ROLLGCNTRVLDNEWDATA (0) ROLLGCNTRVLDOLDDATA (1) ROLLGCNTRINVLD (2) |
| Affected static and global variables | Reads or writes following variables in sub-functions |
|  | PrevRollgCntrRxd |
|  | SigGroupSkipCntr |
|  | SigGroupDataSrc |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | RollgCntrSeqChk |
| External Interface call | None |
| Called by | Called by following functions |
|  | VldtRollgCntr |

## Sub-Function:  RollgCntrSeqChk

### Hardware Related Design

None

### Software Related Design

| RollgCntrSeqChk |  |
| --- | --- |
| Function scope | Local |
| Description | Checks whether present rolling counter value received on IMC channel falls within the range lower and upper drift limits from anticipated rolling counter. |
| Parameters | Requires following parameters |
|  | PrsntRollgCntr- Present Rolling counter value received for the given Signal Group Type: uint8 Range: 0 to 31 |
|  | AntcptdRollCntr - Anticipated Rolling counter value for the given Signal Group Type: uint8 Range: 0 to 31 |
|  | UpprDriftLim - Drift limit on the lower from Anticipated Rolling counter Type: uint8 Range: 0 to 31 |
|  | LwrDriftLim - Drift limit on the lower side from Anticipated Rolling counter Type: uint8 Range: 0 to 31 |
| Return value | Returns following value |
|  | RollgCntrVld - Returns validity of Rolling counter Type: boolean Range: TRUE - Rolling counter value acceptable FALSE - Rolling counter value not acceptable |
| Affected static and global variables | None |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | VldtRollgCntrAlg |

## Sub-Function:  NoDataHndlg

### Hardware Related Design

None

### Software Related Design

| NoDataHndlg |  |
| --- | --- |
| Function scope | Local |
| Description | Handles No data reception cases for Singal Status, Reset counter, Skip counter and Resync counters |
| Parameters | Requires following parameters |
|  | RxSigExtdSts1 - Extended signal Status value Type: ImcArbnRxExtdSts1 Range: IMCARBNRXEXTDSTS_NEVERRXD (0U) IMCARBNRXEXTDSTS_MISS (1U) IMCARBNRXEXTDSTS_DATAINVLD (2U) IMCARBNRXEXTDSTS_PREVDATA (3U) IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP (4U) IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP (5U) IMCARBNRXEXTDSTS_STRTG (6U) Other values are invalid |
|  | RxDataSrc - Imc Channel ID Type: ImcArbnRxDataSrc1 Range: IMCARBNRXDATASRC_PRIM (0U) IMCARBNRXDATASRC_SECDRY (1U) IMCARBNRXDATASRC_NOSRC (2U) All other values are invalid |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | PrimSrcNoDataRxd - No data Received on Primary Source flag Type: boolean TRUE – No data received FALSE – Data received |
|  | SecdrySrcNoDataRxd - No data Received on Secondary Source flag Type: boolean TRUE – No data received FALSE – Data received |
| Return value | void |
| Affected static and global variables | Read and Write to following variables in sub-functions |
|  | SigGroupSkipCntr |
|  | PrimSrcResyncCntr |
|  | SecdrySrcResyncCntr |
|  | PrimSrcRollgCntrResync |
|  | SecdrySrcRollgCntrResync |
|  | ResyncOnPrimActv |
|  | ResyncOnSecdryActv |
|  | RxdSigDataExtdSts |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | Calls following local function |
|  | EvlSigGroupNeverRxdMissSts |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnRx |

## Sub-Function:  EvlSigGroupNeverRxdMissSts

### Hardware Related Design

None

### Software Related Design

| EvlSigGroupNeverRxdMissSts |  |
| --- | --- |
| Function scope | Local |
| Description | Evaluates Signal missing / never received cases and updates signals status. |
| Parameters | Requires following parameters |
|  | RateGroup - Rate Group Id provided by IMC Arbitration Configuration Type: uint8 Range: IMCARBN_RATEGROUPID2MILLISEC_CNT_U08 (0U) IMCARBN_RATEGROUPID10MILLISEC_CNT_U08 (1U) IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U) All other values are invalid |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
| Return value | Returns following value |
|  | RxSigExtdSts1 - Extended Signal Status Type: ImcArbnRxExtdSts1 Range: IMCARBNRXEXTDSTS_NEVERRXD (0U) IMCARBNRXEXTDSTS_MISS (1U) IMCARBNRXEXTDSTS_DATAINVLD (2U) IMCARBNRXEXTDSTS_PREVDATA (3U) IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP (4U) IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP (5U) IMCARBNRXEXTDSTS_STRTG (6U) All other values are invalid |
| Affected static and global variables | Reads / Writes to following variables in functions |
|  | SigGroupNeverRxd |
|  | SigGroupMissCntr |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | Accesses following configuration data |
|  | MISSSIGGROUPTIOUT_CNT_U08 |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnRx |

## Sub-Function:  OvrdSigStsDurgStrtUp

### Hardware Related Design

None

### Software Related Design

| OvrdSigStsDurgStrtUp |  |
| --- | --- |
| Function scope | Local |
| Description | Sets signal status as STARTING in the Signal Status buffer when initial start-up timer is still running. |
| Parameters | Requires following parameters |
|  | SigGroup - Signal Group Id provided by IMC Arbitration Configuration Type: uint8 Range: 0 to 255 |
|  | IniTiOutChkCmpl - Initial time-out completion status Type: boolean TRUE – Time-out completed FALSE – Time-out not completed |
| Return value | void |
| Affected static and global variables | Writes to following variables in functions |
|  | RxSigDataExtdSts |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | Accesses following configuration data |
|  | SIGGROUPCONFIG_REC |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnRx |

## Sub-Function:  GetImcSigSts

### Hardware Related Design

None

### Software Related Design

| GetImcSigSts |  |
| --- | --- |
| Function scope | Local |
| Description | Remaps Extended signal status value to basic version of Signal Status |
| Parameters | Requires following parameters |
|  | SigExtdSts - Extended Signal Status Type: ImcArbnRxExtdSts1 Range: IMCARBNRXEXTDSTS_NEVERRXD (0U) IMCARBNRXEXTDSTS_MISS (1U) IMCARBNRXEXTDSTS_DATAINVLD (2U) IMCARBNRXEXTDSTS_PREVDATA (3U) IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP (4U) IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP (5U) IMCARBNRXEXTDSTS_STRTG (6U) All other values are invalid |
| Return value | Returns following value |
|  | RetSts - Remapped basic Signal Status Type:ImcArbnRxSts1 Range: IMCARBNRXSTS_NODATA (0U) IMCARBNRXSTS_VLD (1U) IMCARBNRXSTS_INVLD (2U) All other values are invalid |
| Affected static and global variables | None |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | GetSigImcData_<DATA_TYPE> |

## Sub-Function:  NoDataRxd

### Hardware Related Design

None

### Software Related Design

| NoDataRxd |  |
| --- | --- |
| Function scope | Local |
| Description | Evaluates no data received condition |
| Parameters | Requires following parameters |
|  | StrtByte - Start byte of given Signal Group Type: uint8 Range: 0 to 255 |
|  | EndByte - End byte of given Signal Group Type: uint8 Range: 0 to 255 |
| Return value | Returns following value |
|  | RetVal- Indicates whether No data received Type: boolean Range: TRUE - No data received FALSE - Data Received |
| Affected static and global variables | None |
| Affected inter-runnable variable | None |
| Exclusive area access | None |
| Configuration access | None |
| Calibration data access | None |
| Called local functions | None |
| External Interface call | None |
| Called by | Called by following functions |
|  | ImcArbnRx |

# Timing / Execution Constraints

## Rationale / Comments

• ImcArbnPer1, ImcArbnPer2 and ImcArbnPer3 periodic functions shall be called prior to calling respective primary and secondary signal source transmit layer functions.

• ImcArbnPer4, ImcArbnPer5 and ImcArbnPer6 periodic functions shall be called after calling respective primary and secondary signal source receive layer functions.

• ImcArbn Receive periodics (ImcArbnPer4, ImcArbnPer5 and ImcArbnPer6) functions shall run prior to  the ImcArbn Transmit periodic functions ((ImcArbnPer1, ImcArbnPer2 and ImcArbnPer3).

## Rates and State Execution

| Sub-Function | Rate (ms) | Cold Init | Warm Init | Operate | Disable |
| --- | --- | --- | --- | --- | --- |
| ImcArbnPer1 | 2 | Req | Req | Req | Req |
| ImcArbnPer2 | 10 | Req | Req | Req | Req |
| ImcArbnPer3 | 100 | Req | Req | Req | Req |
| ImcArbnPer4 | 2 | Req | Req | Req | Req |
| ImcArbnPer5 | 10 | Req | Req | Req | Req |
| ImcArbnPer6 | 100 | Req | Req | Req | Req |

# Serial Communications Interfaces

None

# Additional Information

## Imc Arbitration – Configuration

### Hardware Related Design

None

### Software Related Design

#### Imc Signals

A Signal Group is a basic element of communication for the Imc Arbitration component.

Signals that are need to be transmitted by IMC shall be configured under any of the Signal Group based on their transmit rate requirements.

Following are the general design assumptions about the signals that are transmitted by Imc Arbitration:

- All IMC signals are accessible via RTE to pack it in a particular Signal Group.

- Data type of the signals that are transmitted by Imc Arbitration can be anyone of the following: boolean, uint8, sint8, uint16, sint16, uint32, sint32 and float32. No other types of signals are supported.

#### This component defines unique LOCAL constants for each of the configured signals as Signal ID. Following format is used to define the Signal ID.

#### Format:

#### IMCARBN_<signalname>_CNT_U16

#### signalname – name of the signal in upper case as defined in Imc Arbitration configuration.

#### Signal Group (SG)

Following elements forms a Signal Group:

Pattern ID:  The pattern identification is used as a start of frame to synchronize data on asynchronous communication protocols such as SCI.  As shown in the below figure, the pattern ID is 3 bits (101b).

Rolling Counter:  The rolling counter is used to monitor for new data.  If the rolling counter has been updated, the data is new.  If the rolling counter maintains the same value, the data is old. As shown in the below picture, the Rolling Counter is 5 bits.

Signal Group ID:  The message ID is used to identify the received data group. As shown in the below picture the Message Id is 8 bits.  All data bytes (0 –3) have the same message ID.

Data Byte 0 – 3:  Contains packed data of all signals configured for a given Signal Group.

CRC:  The Cyclic Redundancy Check is calculated on the message ID and all of the data bytes.  This check ensures the integrity of the information being transmitted.  The pattern ID and rolling counter are inherently protected by comparing to the complement, and therefore do not need CRC protection. As shown in the figure 2, the CRC is 8 bits.

Pattern Id compliment and Rolling Counter Compliment: The pattern identification and rolling counter complements are used to identify the end of frame, and also as a basic data reception check.  If the start of frame (Pattern ID and Rolling Counter) and this frame XOR’d together equal all ones, then the full frame is assumed to have been captured and can be passed on for rolling counter and CRC checks. As shown in the below picture, the Pattern ID Complement is 3 bits and The Rolling Counter Complement is 5 bits.

#### This component defines unique global constant for each of the Signal Groups. Following format is used to define the Signal Groups.

#### Format:

#### IMCARBN_<SigGroupName>_CNT_U08

SigGroupName – Signal Group name in uppercase as defined during configuration

#### Rate Group

Imc System supports transmission / reception of Signal Groups at three different rates: 2ms, 10ms and 100ms. Every Signal Group shall be listed under any one of the Rate Group. All Signal Groups that are configured under given Rate Group processed at the same rate. E.g. a Signal Group configured under 2ms rate is processed in a 2ms periodic.

Following global constant values are defined to identify the Rate Groups in the system:

IMCARBN_RATEGROUPID2MILLISEC_CNT_U08     (0U)

IMCARBN_RATEGROUPID10MILLISEC_CNT_U08   (1U)

IMCARBN_RATEGROUPID100MILLISEC_CNT_U08 (2U)

Below picture depicts data organization for Imc Arbitration Tx and Rx frames

Below picture depicts Configuration Description for the component.

#### Types Definitions for Configuration Data

|  | Description |
| --- | --- |
| typedef uint32 (*GetSigVal)(void); | Typedef for the function pointer that will read data from the source FDD. Note that the return value is always considered as (NOT type-casted) uint32 in order to reduce complexity in Tx/Rx buffer handling. |
| typedef struct { GetSigVal GetSigValOper; uint16 SigId_u16; uint8 DestSize_u08; uint8 StrtLocn_u08; }SigPrmRec1; | Structure that defines properties of a signal that need to be communicated across two microcontrollers. GetSigValOper – Pointer to the wrapper function which returns current value of the signal. SigId_u16 – Signal Id – Unique ID generated for the signal by Imc Arbitrator. Throughout the system, this would be used to get value of particular signal. DestSize_u08 – Number of bits used for the signal to pack it in its Signal Group. StrtLocn_u08 - Position of the signal in its Signal Group. |
| typedef struct { SigPrmRec1 const *SigPrm; uint8 NrOfSig_u08; uint8 SigGroupId_u08; boolean PrimSrcOnlySigGroup_logl; }SigGroupRec1; | Structure that defines properties of a Signal Group (SG). SigPrm – Pointer to an array of SigPrm_Rec1 type structures. This holds reference to all signals that are configured under particular Signal Group. NrOfSig_u08 – Number of signals that are packed into particular Signal Group. SigGroupId_u08 - Unique ID generated for the Signal Group by Imc Arbitrator. Throughout the system, this would be used to get frame of particular Signal Group. PrimSrcOnlySigGroup_logl– Indicates whether the Signal Group to be transmitted only on Primary channel. TRUE – Primary only Tx Group. FALSE – Both Primary and Secondary Tx Group. |

#### Configuration Structure: SIGGROUPCONFIG_REC

This configuration is an array of structures of type SigGroupRec1 containing all Imc configuration data for each configured Signal Groups which will be generated during build time based on the configuration.

Each array element of this structure corresponds to a Signal Group configured. Positon of each Signal Group in this structure will be based out of Rate Group: First Signal Groups configured under 2ms Rate Group, then Signal Groups configured under 10ms Rate Group and finally Signal Groups configured under 100ms Rate Group.

Sample configuration:

/* General configuration for signal group TestSigGroup1 */

const SigGroupRec1 SIGGROUPCONFIG_REC[IMCARBN_TOTALNROFSIGGROUP_CNT_U08] =

{

{

&TESTSIGGROUP1[0], /* Signal group config structure for TestSigGroup1 signal group */

1U,/* Number of signals configured in the signal group */

IMCARBN_SGTESTSIGGROUP1_CNT_U08, /* Signal Group identifier */

TRUE, /* Primary Only Signal group */

},

{

&ImcArbn_ TESTSIGGROUP2[0], /* Signal group config structure for TestSigGroup2 signal group */

1U,/* Number of signals configured in the signal group */

IMCARBN_SGTESTSIGGROUP2_CNT_U08, /* Signal Group identifier */

FALSE, /* Primary Only Signal group */

},

{

&ImcArbn_ TESTSIGGROUP3[0], /* Signal group config structure for this signal group */

5U,/* Number of signals configured in the signal group */

IMCARBN_SGTESTSIGGROUP3_CNT_U08, /* Signal Group identifier */

FALSE, /* Primary Only Signal group */

},

};

#### Configuration Structure: <Signal Group Name>

This configuration array of structure of type SigPrmRec1 contains Imc configuration data for individual Signal Groups which will be generated during build time based on the configuration.

Individual elements in this array correspond to properties of each Signal configured under given Signal Group.

Following naming convention will be followed while generating Signal Group structures:

<SIGGROUPNAME>_REC

Sample Configuration:

/* Signal configuration for the signal group TestSigGroup1 */

STATIC const SigPrmRec1 TESTGIGGROUP1_REC[] =

{

{

&GetTestSig1, /* Pointer to the function which provides Signal data */

IMCARBN_TESTSIG1_CNT_U08, /* Signal Id */

8U,/* Number of bits to be used in the Signal Group for this signal */

0U, /* Position of the signal from LSB */

},

{

&GetTestSig2, /* Pointer to the function which provides Signal data */

IMCARBN_TESTSIG2_CNT_U08, /* Signal Id */

8U,/* Number of bits to be used in the Signal Group for this signal */

8U, /* Position of the signal from LSB */

},

{

&GetTestSig3, /* Pointer to the function which provides Signal data */

IMCARBN_TESTSIG3_CNT_U08, /* Signal Id */

16U,/* Number of bits to be used in the Signal Group for this signal */

16U, /* Position of the signal from LSB */

},

};

#### Signal data access functions: Get<Signal Name>

This component will generate a Signal data access function for every IMC Signal configured in the configuration.

Irrespective of configured Signal data type, this function will return uint32 value.

Prototype:

uint32 GetSignalName> (void) ;

This function will read data for configured Signal from respective application component.

Signals of type uint32 will be returned without any typecasting.

Boolean, uint8, sint8, uint16, sint16 signals will be typecasted to uint32 and typecassted value will be returned.

Float32 signals will be read as uint32 without typecasting and will be returned as uint32 (i.e this means underlying float data format will not be disturbed).

Sample Function 1: TestSig1 is a float signal

/* Wrapper function to get value of TestSig1 */

uint32  GetTestSig1(void)

{

uint32 RetVal_Uls_T_u32;

float32 SigVal_Uls_T_f32;

SigVal_Uls_T_f32 = Rte_Read_TestSig1_Val ();

RetVal_Uls_T_u32 =  *((uint32*)&SigVal_Uls_T_f32);

return(RetVal_Uls_T_u32);

}

Sample Function 2: TestSig2 is a boolean signal

/* Wrapper function to get value of TestSig2 */

uint32  GetTestSig2(void)

{

uint32 RetVal_Uls_T_u32;

RetVal_Uls_T_u32 = Rte_Read_TestSig2_Val ();

return(RetVal_Uls_T_u32);

}

#### Configuration Array: RATEGROUPOFFS_CNT_U08

This configuration array provides offset for accessing for each Rate group configuration data in the configuration structure ImcArbn_SigGroupConfig_Rec. It is defined as follows:

RATEGROUPOFFS_CNT_U08 [IMCARBN_NROFRATEGROUP_CNT_U08] = {   0U,                                                                                                         IMCARBN_NROF2MILLISECRATEGROUP_CNT_U08,                                                                                                         IMCARBN_NROF2MILLISECRATEGROUP_CNT_U08 + IMCARBN_NROF10MILLISECRATEGROUP_CNT_U08 };

#### Configuration Array: NRSIGGROUPINRATEGROUP_CNT_U08

This configuration array provides number of Signal group defined for each Rate Group in the configuration structure ImcArbn_SigGroupConfig_Rec. It is defined as follows:

NRSIGGROUPINRATEGROUP_CNT_U08 [IMCARBN_NROFRATEGROUP_CNT_U08] = {   IMCARBN_NROF2MILLISECRATEGROUP_CNT_U08,                                                                                                                     IMCARBN_NROF10MILLISECRATEGROUP_CNT_U08,                                                                                                                      IMCARBN_NROF100MILLISECRATEGROUP_CNT_U08 };

#### Configuration Array: MISSSIGGROUPTIOUT_CNT_U08

This configuration array provides missing count value for individual Rate Groups. Count values are corresponding to be number of cycles of respective Rate Groups. It is defined as follows:

MISSSIGGROUPTIOUT_CNT_U08[IMCARBN_NROFRATEGROUP_CNT_U08] = { MISSTHD2MILLISECRATEGROUP_CNT_U08,                                                                                              MISSTHD10MILLISECRATEGROUP_CNT_U08,                                                                                               MISSTHD100MILLISECRATEGROUP_CNT_U08

}

## Imc Signal Mapping for Receiving FDDs

### Hardware Related Design

None

### Software Related Design

Since configured Imc Signal names may not match with the signal name which is used in by receiving FDDs, Signal mapping mechanism is provided to resolve mapping between IMC input signals and user of IMC input signals.

FDDs which needs signal from Imc Arbitration component shall use following naming convention for naming Signal ID ( which is an argument for GetSigImcData_xxx):

Format:

IMCARBN_<componentshortname><signalname>_CNT_U16

componentshortname – Short name of the component which is interested in receiving the signal from Imc Arbitration.

signalname – name of the signal as defined by the receiving component.

Note that ownership of this constant is still with Imc Arbitration (hence the “IMCARBN_” prefix).

Imc Arbitration component will generate configurations to map these Signal IDs with its own Signal IDs for each of the Imc signal.

e.g.

IMCARBN_FDD1OPTIONALSIGNAME1_CNT_U16

## Enumerations - Signal Status, Extended Signal Status and Signal Source

### Hardware Related Design

None

### Software Related Design

This component uses three RTE enumerations and one local enumeration:

- Extended Signal Status (ImcArbnRxExtdSts1): Following table indicates list of values and its meaning.

| Extended Signal Status | Meaning |
| --- | --- |
| IMCARBNRXEXTDSTS_STRTG | Initial Start-up time not completed |
| IMCARBNRXEXTDSTS_DATAVLDWITHBACKUP / IMCARBNRXEXTDSTS_DATAVLDWITHNOBACKUP | Valid Data Received |
| IMCARBNRXEXTDSTS_NEVERRXD | Message never received |
| IMCARBNRXEXTDSTS_DATAINVLD | Invalid - CRC, Rolling Counter |
| IMCARBNRXEXTDSTS_PREVDATA | Message Missing - Threshold yet to be reached |
| IMCARBNRXEXTDSTS_MISS | Message Missing - Threshold reached |

Extended signal status value would be returned to the caller when the Server Runnable GetSigImcDataExtdSts invoked.

- Signal Status (ImcArbnRxSts1): Following table indicates list of values and its meaning.

| Signal Status | Use case |
| --- | --- |
| IMCARBNRXSTS_NODATA | Initial Start-up time not completed |
| IMCARBNRXSTS_VLD | Valid Data Received |
| IMCARBNRXSTS_INVLD | Message never received, Invalid CRC, Invalid Rolling Counter, Message missing |

Signal status value would be returned to the caller when the Server Runnable GetSigImcData invoked.

- Signal Data Source (ImcArbnRxDataSrc1): Following table indicates list of values and its meaning.

| Signal Data Source | Meaning |
| --- | --- |
| IMCARBNRXDATASRC_PRIM | Signal received on Primary Source |
| IMCARBNRXDATASRC_SECDRY | Signal received on Secondary Source |
| IMCARBNRXDATASRC_NOSRC | Signal is not received on any of the sources. |

Signal data source would be returned to the caller when the Server Runnable GetSigImcDataExtdSts invoked.

- Rolling counter status (ImcArbnRxRollgCntrSts1):	This enum used to indicate status of Rolling counter evaluation:

| Signal Data Source | Meaning |
| --- | --- |
| ROLLGCNTRVLDNEWDATA | Rolling counter valid with New data |
| ROLLGCNTRVLDOLDDATA | Rolling counter valid with old data |
| ROLLGCNTRINVLD | Rolling counter invalid. |

# Revision Record & Change Approval

| Rev | Date | Change Control # | Change Description |
| --- | --- | --- | --- |
| 1 | 15/01/17 | EA4#9025 | Initial Version |
| 2 | 17/01/17 | EA4#9025 | Updates: 1. Tx flow chart to remove ternary operator in the Rolling counter calculation 2. SetRxSigGroup – Data type for Buf changed from uint8* to Ary1D_u8_8* |